SECTION 02481 - GRASS PLANTING

PART 1 - GENERAL

1.01	GENERAL PROVISIONS:  The General Conditions and Special Conditions contained in this IFB shall govern this section of the work.

1.02	WORK INCLUDED:  Work includes all labor, materials, equipment and incidentals required to complete the grassing by a licensed landscape contractor as shown on the plans and as herein specified.

1.03	REFERENCE:  DPR Standard Specifications for Landscaping, August 1989.

1.04	EXPLANATION AND PRECEDENCE OF DRAWINGS

A.	For purposes of clarity or legibility, drawings are essentially diagrammatic and although the locations and areas of plant material are drawn to scale, Contractor shall make use of all data in all contract documents and shall verify this information at the site.

B.	All work called for on the drawings by notes shall be furnished and installed whether or not specifically mentioned in the specifications.

1.05	WORK SPECIFIED IN OTHER SECTIONS

A.	Stripping surface, stockpiling and spreading of existing on-site soil and/or furnishing and spreading of imported screened soil in all planting areas except for trees and shrubs.

1.06	SOIL PREPARATION (for grass and ground cover planting)

A.	General

1.	These specifications shall supersede Sections 50 and 51 "Standard Specifications for Public Works Construction," September 1986, City and County of Honolulu.

2.	Soil Testing:  Complete soil testing and soil analysis shall be done on all existing on-site soil and/or imported screened soil to be used for landscaping on the site.  Soil testing shall be done by the University of Hawaii Cooperative Extension Service or an independent soil engineering laboratory.  Soil samples shall be tested for pH and "available" phosphorous, potassium, calcium and magnesium.  A test for salinity may be requested upon inspection by the Construction Manager.  Contractor shall bear all cost of soil testing.

B.	Materials

1.	Commercial fertilizer shall be of the analysis 10-30-10, or as determined by the Construction Manager.

2.	Lime (calcium carbonate) shall be agricultural type acceptable to the Construction Manager.

3.	Gypsum (calcium sulfate) shall be agricultural type acceptable to the Construction Manager.

4.	Organic nutrient soil conditioner (humus) shall be Ferto (6-4-2) or Gro-Power Plus (5-3-1) or equal acceptable to the Construction Manager.

5.	Organic non-nutrient-soil conditioner shall be “Menehune Mulch” or Big "R" Redwood soil conditioner or equal acceptable to the Construction Manager.

C.	Soil Amending Application Rates

1.	Organic nutrient soil conditioner shall be applied at the rate of l50 lbs. per l,000 square feet or as specified on plans.

2.	Organic non-nutrient-soil conditioner shall be applied as specified on plans.

3.	Addition of amendments:  Addition of commercial fertilizer and amendments shall be in strict accordance to specifications and as determined by the Construction Manager after submission of soil test report.

4.	Contractor shall submit copies of all purchase and delivery receipts to the Construction Manager for all amendments ordered and used on this project.

D.	Soil Preparation Procedure

1.	After site grading operations have been completed, shape and fine grade to a smooth fine surface the entire site in accordance with the grading plan.

2.	Spread amendments on ground in even layers at rates specified in Section 1.06.C.1 and 2 and incorporate to a depth of 4 inches by rotary hoe method, working over all areas in alternate directions.  ALL AMENDMENTS SHALL BE SPREAD OVER THE SITE IN THE PRESENCE OF AND ACCEPTED BY THE CONSTRUCTION MANAGER BEFORE INCORPORATION.  This soil amending and acceptance procedure shall not be deleted, substituted nor amended in any manner.

3.	Drag all areas by mechanical means to break up clods.  Remove debris and rocks over 1 inch in diameter by use of a spring tooth harrow with debris screen container.  Fine grade to a smooth continuous surface and roll with a lightweight disc roller prior to planting.

1.07	GRASS PLANTING

A.	General

1.	The work covered under this section shall include the furnishing of all labor, equipment, materials and incidentals necessary for grass planting or grass restoration, complete in place, in strict accordance with this section of the specifications and applicable drawings.

2.	Grass planting shall commence immediately after the soil preparation has been completed and accepted by the Construction Manager.

3.	The Contractor may elect to plant grass by either seeding, stolonizing or hydromulching, unless otherwise indicated on the drawings.

B.	Materials

1.	Grass shall be common Bermuda, cynodon dactylon, unless otherwise indicated on the drawings.

2.	Grass seed shall be freshly hulled and meet the following requirements:

Pure Seed	95.0% minimum

Crop Seed	1.0% maximum

Weed	0.5% maximum

Inert Material	5.0% maximum

Germination	85.0% minimum

Grass seeds shall be delivered to the site in unopened, sealed container, labeled with brand name and percent purity.  Labeling shall indicate that the seeds passed a certified germination test no more than 12 months prior to use.

3.	Grass stolons shall be healthy, living runners 2 inches to 4 inches long with a well-developed root system.  They shall be freshly cut and free from weeds.  Immediately following harvesting, the stolons shall be thoroughly watered then covered and kept moist during storage and transportation until planted.

C.	Procedure

1.	Preparation of planting area:  Refer to Section 1.06, Soil Preparation.

2.	Planting by seed:  Ground surface shall be raked to a smooth even plane, broadcast seeds uniformly by mechanical sower at a minimum rate of 5 lbs. per 1,000 square feet.  Sow one-half the seeds in one direction and the remainder at right angles to the first direction.  The seeds shall be incorporated into the top 1/4 inch to 1/2 inch of soil.  After sowing, the seeded area shall be compacted by hand or a cultipacker or mechanical roller weighing 60 to 90 pounds per lineal foot of roller.  The area shall be thoroughly and gently watered and kept moist during the germination period.

3.	Planting by stolons:  After ground surfaces have been raked to a smooth and even plane, broadcast stolons uniformly by hand at a minimum rate of 6 bushels per 1,000 square feet.  The stolons shall be incorporated into the top 1/4 inch to 1/2 inch of soil.  After broadcasting, the planted area shall then be disked and rolled by a mechanical roller weighing 60 to 90 lbs. per lineal foot of roller to insure close contact of the soil and stolons.  Do not roll on wet soil.

4.	Planting by hydromulching

(a)	Description:  This work shall consist of furnishing and applying by hydromulching, seeds or stolons, and mulch to areas designated on the plans or ordered by the Construction Manager and shall include continuous care and maintenance in accordance with these specifications.  Such areas shall include those damaged or made barren by construction operations or equipment.  All steep banks 3 to 1 slope or more shall be hydromulched for erosion control.

(b)	Materials

(1)	Seeds:  Seeds shall be hulled common Bermuda (cynodon dactylon), certified, meeting the following requirements:

Pure Seed	95.0% minimum

Crop Seed	1.0% maximum

Weed	0.5% maximum

Inert Material	5.0% maximum

Germination	85.0% minimum

The seeds shall be applied at the rate of 50 pounds per acre (minimum) and within 12 months of the date of the certified germination test.

(2)	Stolons:  Shall be healthy, living, well-developed runners, 1 inch to 2-1/2 inches in length, freshly dug up and free from weeds.  Immediately following harvesting, the stolons shall be thoroughly watered then kept moist during storage and transportation until planted.  Stolons shall be applied at the rate of 8 bushels per 1,000 square feet (minimum).

(3)	Mulch:  Mulch shall be specifically processed fiber containing no growth or germination inhibiting factors.  It shall be such that after addition and agitation in the hydraulic equipment with seeds, fertilizer, water and other additives not detrimental to plant growth, the fibers will form a homogeneous slurry.  When hydraulically sprayed on the soil, the fibers shall form a blotter-like ground cover which readily absorbs water and allows infiltration to the underlying soil.  In every application, complete coverage of the soil shall be attained.  Mulch shall be applied at the minimum rate of 1,700 pounds per acre.

(c)	Procedure

(1)	The Contractor shall notify the Construction Manager not less than 24 hours in advance of any hydromulching operation and shall not begin work until area for hydromulching has been prepared and accepted by the Construction Manager.  Acceptance shall include inspection of fine grading to provide for the collection and disposal of surface and subsurface water to protect planting areas against unnecessary erosion.  Acceptance shall not relieve the Contractor of his responsibility to restore any damage to the grade until the planted areas are accepted.  Following the Construction Manager’s acceptance, hydromulching of the approved planting areas shall begin promptly.

The hydromulch equipment shall be capable of mixing all the necessary ingredients to a uniform mixture and to apply the slurry to provide uniform coverage.  Seeds, fertilizer and mulch mix shall be applied in one operation by approved hydraulic equipment.  The equipment shall have a built-in agitation system with an operating capacity sufficient to keep the mix in uniform distribution until pumped from the tank.  Distribution and discharge lines shall be large enough to prevent stoppage and shall be equipped with hydraulic discharge spray nozzles which provide a uniform distribution of the slurry.  Areas inaccessible to hydromulching application shall be stolonized or seeded by hand broadcast method acceptable to the Construction Manager and fertilized.

Water shall be applied immediately following mulching in such quantities as to moisten the soil and mulch.  Watering shall be continued in such manner, quantity and frequency to insure proper germination and growth and shall be done in a way that will prevent erosion.  Watering equipment shall be of a type that will not cause erosion or runoff.  If method is deemed unacceptable by the Construction Manager, it shall be corrected by the Contractor.  Should the slope erode or silt be transported to pose as potential pollutants, it shall be immediately removed and prevented from entering into water channels.  All eroded areas shall be restored to the original grade as soon as possible.

1.08	PLANTING MAINTENANCE, INSPECTION AND ACCEPTANCE

A.	General:  This item of work shall consist of the maintenance of all plants and planted areas in optimum growing condition and appearance and the inspection and acceptance of landscape planting.

B.	Pre-Maintenance Inspection

1.	A pre-maintenance inspection shall be held at the completion of all planting operations and prior to the beginning of the formal maintenance period.

2.	All deficiencies shall be corrected and all plantings accepted by the Construction Manager before the issuance of a commencement date of the formal maintenance period.

C.	Period of Maintenance

1.	Maintenance shall begin immediately after completion and acceptance of all planting and shall continue for 60 calendar days thereafter, unless otherwise specified in the Proposal or on the drawings.  The care of plants prior to completion of all planting, including relocated trees, shall not be considered as part of the maintenance period but only as an incidental to landscape work.

2.	Maintenance shall include watering, weeding, fertilizing, topdressing with amended imported screened soil acceptable to the Construction Manager, mowing, spraying for insects, replanting and any other work necessary to maintain all plants in a healthy growing condition.  The Contractor shall be responsible for the protection of all plants and planting areas during the maintenance period and shall pay for all water required during the maintenance period unless otherwise specified.  All dead plant materials shall be replaced immediately.  During the last week of the maintenance period, the Contractor shall fertilize the entire planting areas as follows:

Organic nutrient-soil conditioner (humus) at the rate of 25 lbs. per 1,000 square feet.  Ferto (6-4-2) or Gro-Power Plus (5-3-1) shall be applied in the presence of the Construction Manager.

D.	Final Inspection and Acceptance

1.	At the completion of all planting operations and the maintenance period, an inspection shall be performed.

2.	The Contractor shall request the inspection in writing to the Construction Manager 7 calendar days prior to the completion of the maintenance period in order that a mutually agreeable time for inspection may be arranged.

3.	The Contractor and the Construction Manager, or their representatives, shall be present at the inspection.

4.	Any plant material found not to be in healthy growing condition shall be immediately replaced by the Contractor.

5.	Acceptance of the grass planting after the maintenance period shall be contingent upon the following:

a.	95% coverage of the overall area shall be required for grass planting.

b.	All germinated areas shall be healthy and living at the end of maintenance period.

c.	Weeds shall not exceed an area greater than 10% of the overall grass area.

d.	Grass shall be mowed and not taller than 1 inch in height.  All clippings shall be removed.

e.	All depressions shall be filled to proper grade and area regrassed as required.

6.	If, at the final inspection, the Construction Manager is of the opinion that all or certain portions of the work are not acceptable as to the intent of the drawings and specifications, an additional 30 days maintenance period shall be extended at no cost to the State.  During this period, the Contractor shall meet all requirements and correct all deficiencies, including any additional mowing and fertilizing.  After the 30 days, if the work is still unacceptable or does not meet the conditions as specified under Section 1.08.D.4 and 5, Planting Maintenance, Inspection and Acceptance, the Contractor shall be charged with liquidated damages for each and every calendar day thereafter and shall be based on the amount in Section SC-05 of the Special Conditions.

7.	If all plant materials are accepted at this inspection by the Construction Manager, the Contractor shall be relieved of further maintenance.

END OF SECTION