DIVISION 2 - SITE WORK

SECTION 02100 –GRUBBING

PART 1 – GENERAL

  - GENERAL CONDITIONS:  The General Conditions and Special Conditions contained in this IFB shall govern this section of the work.

  - WORK INCLUDED

- Furnish all labor, materials, equipment and tools necessary to accomplish all grubbing of vegetation and roots as indicated on the plans and as specified herein.

- It shall be the responsibility of the Contractor to examine the project site and determine for himself the existing conditions.

- Obvious conditions of the site existing on the date of the bid opening shall be accepted as part of the work, even though they may not be clearly indicated on the plans and/or described herein or may vary therefrom.

- All debris of any kind accumulated from grubbing shall be disposed of off-site weekly and the whole area left clean.  The Contractor shall be required to make all necessary arrangements related to the proposed place of disposal.

- Burning onsite will not be permitted.

PART 2 – PRODUCTS (NOT USED)

PART 3 – EXECUTION

  - SEQUENCE OF WORK:  All sequence of work shall be subject to the approval of the Construction Manager.

  - PROTECTION

- Adequate precautions shall be taken before commencing and during the course of the work to insure the protection of life, limb and property.

- The Contractor shall protect from damage all surrounding structures, trees, plants, grass, walks, pavements, utility boxes, mail boxes etc.  Any damages will be repaired or replaced by the Contractor to the satisfaction of the Construction Manager.

  - PERMITS:  The Contractor shall apply for and obtain the necessary permits prior to the commencement of work.  The Contractor shall pay for all fees.

  - BARRICADE:  Erect temporary barricade to prevent people and animals from entering the project area, to the extent as approved by the Construction Manager.  Such barricades shall not be less than 5’-0” in height.  The extent of barricades may be adjusted as necessary with the approval of the Construction Manager.  This work shall be accomplished to the satisfaction of the Department and at no extra cost to the Department.  Barricades shall be removed upon completion of work and job site premises left clean.

  - MAINTAINING TRAFFIC

- The Contractor shall conduct operations with minimum interference to streets, driveways, sidewalks, and provide access to the existing mail boxes.

- When necessary, the Contractor shall provide, erect and maintain lights, barriers, etc., as required by traffic and safety regulations with special attention to protection of life.

  - CONSTRUCTION LINES, LEVELS AND GRADES

- The Contractor shall verify all lines, levels and elevations indicated on the plans before any clearing, excavation or construction begins.  Any discrepancy shall be immediately brought to the attention of the Construction Manager and any change shall be made in accordance with his instruction.  The Contractor shall not be entitled to extra payment if he fails to report the discrepancies before proceeding with any work whether within the area affected or not.

- All lines and grades shall be established by a Surveyor licensed in the State of Hawaii.

  - GRUBBING

- The Contractor shall grub the ground surface within the area to be graded of all mulch, grass and weeds to 4 inches below present grades.

- No roadway excavation or trench excavation shall be undertaken until area has been grubbed.

  - CONTRACT ZONE LIMITS:  The Contract Zone Limits shown on the plans indicate only in general the limits of the work involved.  The Contractor, however, is required to perform any and all necessary and incidental work which may fall outside of these demarcation lines.

  - VERIFICATION OF EXISTING GRADES:  Verify existing grades, inverts, and improvements before any grubbing work is done.  Immediately bring to the attention of the Construction Manager any discrepancy, and make any changes in accordance with his instructions.  Starting of grubbing operations will be construed to mean that the Contractor agrees that the existing grades, inverts, and improvements are essentially correct as indicated.  No extra compensation will be allowed if existing grades, inverts, and improvements are in error after verification thereof or if he fails to report the discrepancies before proceeding with any work.

  - CLEAN-UP:  Clean up and remove all debris accumulated from construction operations from time to time, when and as directed by the Construction Manager.  Upon completion of the construction work and before final acceptance of work, remove all surplus materials, equipment, etc., and leave entire job site clean and neat.

END OF SECTION