SECTION 02721 – STORM DRAINAGE SYSTEM

PART 1 – GENERAL

  - GENERAL CONDITIONS:  The General Conditions and Special Provisions preceding these specifications shall govern this section of the work.

  - WORK INCLUDED:  Furnish all labor, materials, tools, equipment and related items necessary to complete, in place, the storm drainage system in conformity with the dimensions, profiles, sections, and details shown on the plans.  Work relating to drainpipes and drainage structures shall be governed by the following sections of the Standard Specifications:

Trench Excavation and Backfill	Section 11

Drainpipes	Section 24

Drain Manholes	Section 25

Catch Basins and Storm Water Inlets	Section 26

Portland Cement Concrete	Section 39

Concrete Structures	Section 40

  - CONTRACTOR SUBMITTALS:  Shop drawings shall be submitted for precast manholes, catch basins and storm water inlets.

PART 2 – PRODUCTS

  - MATERIALS

- Drainpipe:  Reinforced Concrete Pipe, Class III, AASHTO M170.

- Materials for the storm drainage system shall be in accordance with the sections of the Standard Specifications noted hereinbefore.

PART 3 – EXECUTION

  - INSTALLATION:  Install the storm drainage system in accordance with the sections of the Standard Specifications noted hereinbefore.

  - The Contractor shall be responsible for precisely laying out the storm drain line shown on the contract plans.  The location shown on the contract plans of the various existing utility lines which the new lines are to cross over or under or connect to were determined on the basis of the best information available; however, no assurance can be provided that the actual locations will be precisely as shown on the contract plans.

  - In performing all work, the Contractor shall exercise due care and caution necessary to avoid any damage to and impairment in the use of any existing utility lines.  Any damage inflicted on existing lines resulting from the Contractor’s operations shall be immediately repaired and restored as directed by the Engineer at the Contractor’s expense.

END OF SECTION